JLG G15-44A

- Axle-Front: High Bias, Limited Slip

- Axle-Rear: Planetary

- Capacity – Fuel Tank: 38 gal. / 143.85 L

- Drawbar Pull: 24000 lb / 10886.22 kg

- Engine Horsepower: 130 hp / 96.98 kW

- Engine Manufacturer: Cummins

- Engine Model: QSB4.5L ATAAC Tier 4 Interim

- Frame Leveling: 10 Degrees

- Function Pump Type: Load Sensing Piston

- Ground Clearance: 1 ft 6 in. / 0.46 m

- Lift Capacity at Full Height: 7000 lb / 3175.15 kg

- Lift Capacity at Max Reach (Stabs Up): 2000 lb / 907.18 kg

- Load Center: 2 ft / 0.61 m

- Machine Height: 8 ft 5 in. / 2.57 m

- Machine Length: 20 ft / 6.1 m

- Machine Width: 8 ft 5 in. / 2.57 m

- Maximum Engine RPM: 1500 rpm

- Maximum Lift Capacity: 15000 lb / 6803.89 kg

- Maximum Lift Height: 44 ft / 13.41 m

- Maximum Reach: 31 ft / 9.45 m

- Operating Vehicle Weight: 32500 lb / 14741.75 kg

- Outriggers or Stabilizers: No

- Outside Turning Radius: 14 ft 6 in. / 4.42 m

- Service Brake Type: Wet Multi-Disc

- Speed – Maximum Lift: 16 sec.

- Speed – Maximum Lower: 15 sec.

- Speed – Maximum Telescope In: 13 sec.

- Speed – Maximum Telescope Out: 17 sec.

- Speed – Maximum Travel: 20 mph / 32.19 km/h

- Tire Size: 17.5 x 25

- Transmission – Nr Forward Gears Standard: 4

- Transmission – Nr Reverse Gears: 3

- Transmission Type: Powershift

- Wheelbase: 12 ft / 3.66 m